# HAMPSHIRE COUNTY COUNCIL

# ECONOMY TRANSPORT & ENVIRONMENT GROUP

# Determining the Location of Buried Apparatus Prior to Highway Works

This process was agreed at the Highways Traffic and Transport Board on the 7th of January 2015.

HSG47 document states that those undertaking works near underground services should identify their location to minimise risks to safety and to protect the asset. The Highways Authority Utility Committee (HAUC) Code of Practice “Measures Necessary Where Apparatus is Affected by Major Works” describes a process (The C notice process) by which the location of buried apparatus may be determined.

This document seeks to align both National documents to provide a consistent approach to determining the location of buried apparatus.

## Background

## HSE (Health and Safety Executive)

Before any work is undertaken the HSE guidance on “Avoiding danger from underground services” (HSG47) should be considered. This document states “Plans can give an indication to the location, configuration and number of underground services at a particular site…However are not always drawn accurately and to scale even if they claim to be, you should not rely on them to obtain distances or depths” and goes on to state “Plans alone are not sufficient to identify and locate services before starting work. They provide basic information on which to base a thorough survey before work begins”

## C2 Notice process as taken from the code of practice for “measures necessary where apparatus is affected by major works (diversionary works)”

This process is used to determine the location of apparatus and any need to divert such apparatus as a result of major works. (The process is described in the Diversionary Works Code of Practice – New Roads & Street Works Act 1991)..

At the C2 stage “the highway authority would seek from all undertakers, details of their apparatus within a specific section of the scheme which is being considered for improvement without making any commitment to the scheme”

Essentially, a C2 is a preliminary enquiry to an SU for plant location and apparatus details, and can be used to determine plant locations. The New Roads and Street Works Act 1991 section 79 states “All undertakers shall, except in such cases as may be prescribed, record the location of every item of apparatus belonging to him as soon as reasonable practicable”.

## Obtaining the information – The Process

Undertakers should provide such information as they have available from records and draw attention to any likely special problems which could arise from authorities works.

Under section 79.3 of the New roads and Streetworks Act 1991 it states “An undertaker shall make their records available for inspection, at all reasonable hours and free of charge, by any person having authority to execute works of any description in the street or otherwise appearing to the undertaker to have a sufficient interest”. In essence this means that the undertaker has to allow any relevant authority to view their records for free at their premises. They do not however have to supply the authority with copies for free. They may charge for photocopying and sending information.

Using the NRSWA C2 process is considered to be good practice for all works involving excavation.

It is common practice for SU’s to provide their buried apparatus information in contrasting ways. Some will use a third party, such as a website. While others will provide their own info, supplying the relevant authority with an email address or internal website. These websites often require pre-registration; this should be taken into account before trying to submit a C2 request as it can be a time consuming process.

Work promoters should contact all appropriate organisations from the “C2 contacts spread sheet” below (link), to determine the presents of apparatus. There are 60+ organisations with buried apparatus in Hampshire’s highways, many of which are small, private networks, usually providing telecoms services. Accordingly it is understood that contacting all such organisations is likely to be an extremely onerous task and likely to provide little benefit in many instances. Accordingly Works Promoters may instead opt to only determine plant locations for the apparatus which they consider they have a high risk of impacting. Risk should be determined taking into consideration the following factors;

- Risk to safety of striking buried apparatus

- Frequency of works undertaken by the apparatus owner

- Financial and reputational cost to the County Council of damaging the apparatus

- Location of the planned works

In all cases works promoters should contact operators of the following types of network;

- Oil & gas networks

- Electricity / power networks

- Water/ sewage networks

- Major telecoms networks

Owners of apparatus relating to smaller telecoms networks and other low risk apparatus types may be omitted from the search on a case by case basis.

However the information is made available, the Code of Practice states that it should normally be provided within 10 working days, unless otherwise agreed. But works promoters must note that utility companies have no statutory requirement to abide by the timescales indicated in the Code of Practice.

When the highway authority is undertaking works, it is important to not only find the SU apparatus in the area, but also the apparatus that the Authority or other Government organisations are responsible for, such as Drainage, private apparatus, street lighting cables and lamps, areas of archaeological / historical importance, MoD networks etc. This information maybe available on corporate mapping systems or obtained from local area offices. The National Street Gazetteer records will also hold a wealth of information to assist works promoters in determining the possible presence of buried apparatus, structures or other issues that may cause difficulties when undertaking works.

Appendix 3 lists all of the organisations known to have apparatus in the public highway.

# Appendix 1: Requesting information via email

It maybe the case that “C2” information is only available if requested via email. In this situation the following may be used as a template.

“To Whom it may concern,

Please could you provide me with any of your organisations apparatus details for the following location: 123 Example Street, etc……………………

Easting’s…………….Northing’s……….

Kind regards”

It is also preferable if a map can be attached.

# Appendix 2: Contact Details for Locating County Council Highway Authority Apparatus.

Street Lighting Enquiries Countywide = hsl@hants.gov.uk

Traffic (ITS) Equipment Countywide = intelligent.transport.systems@hants.gov.uk

Areas of Archaeological Importance = historic.environment@hants.gov.uk

Highway Drainage & Privately licensed apparatus under Section 50 of NRSWA = ete.hoc@hants.gov.uk

# Appendix 3: Information Pertaining to Utility Co’s Known to Have Apparatus in Hampshire

List correct as of 19/11/2021

| Organisation | Utility Type | Working area/ District | Average No of works (Based on 2020/2021) |
| --- | --- | --- | --- |
| Abovenet Communications UK Ltd1 | Telecom | Basingstoke, Rushmoor | 4 |
| Albion Water Ltd | Water | Only in Knowle | 2 |
| Bournmouth Water2 | Water | New Forest | 0 |
| British Gas Connections3 | Power | All Hampshire districts | 0 |
| BSkyB Telecommunications Services Ltd4 | Telecom | All Hampshire districts | 0 |
| BT Openreach | Telecom | All Hampshire districts | 9520 |
| Cable & Wireless5 | Telecom | All Hampshire districts | 217 |
| Cholderton and District Water | Water | Only present in Shipton Bellinger | 0 |
| City of London Telecomms6 | Telecom | Rushmoor | 16 |
| CityFibre Metro Networks | Telecom | East Hants, Fareham, Havant | 38 |
| Cloud HQ Didcot Fibre | Telecom | Rushmoor | 2 |
| Colt Telecommunications6 | Telecom | Rushmoor | 16 |
| Concept Solutions7 | Telecom | Rushmoor | 1 |
| Energetics8 | Power | All Hampshire districts | 31 |
| Energis Comms9 | Telecom | All Hampshire districts | 0 |
| Energy Assets Networks Limited10 | Various | Test Valley, Basingstoke, Rushmoor | 2 |
| Environment Agency | Other Authority | All Hampshire districts | 1 |
| ES Pipelines | Oil | All Hampshire districts | 11 |
| ESP Connections Ltd3 | Power | Unknown | 0 |
| ESP Electricity | Power | All Hampshire Districts | 4 |
| Esso | Oil | All Hampshire districts | 85 |
| EUNetworks Fibre UK Ltd | Telecom | All Hampshire districts | 0 |
| Fibernet | Telecom | All Hampshire districts | 0 |
| Fibrespan Ltd | Telecom | All Hampshire districts | 0 |
| Fulcrum Pipelines Ltd | Oil / Gas | All Hampshire districts | 18 |
| Gigaclear Ltd | Telecom | East Hants | 6 |
| Giganet Ltd11 | Telecom | East Hants, New Forest | 0 |
| Glide Business Ltd12 | Telecom | All Hampshire Districts | 2 |
| Glide Fibre Ltd7 | Telecom | Rushmoor | 1 |
| Global Crossing (UK) Telecoms Ltd13 | Telecom | All Hampshire districts | 12 |
| Grayshott Gigabit Ltd | Telecom | Operating in Gratshott only | 0 |
| GTC Pipelines Ltd | Oil | All Hampshire districts | 89 |
| H2O Networks | Telecom | All Hampshire districts | 0 |
| Hampshire County Council (Private apparatus, highway drains, ITS equipment and street lighting | N/A | All Hampshire districts | 29340 |
| Hampshire County Council, areas of historical importance | N/A | All Hampshire districts | N/A |
| Hutchison 3G | Telecom | All Hampshire districts | 0 |
| Independent Fibre Networks | Telecom | Unknown | 0 |
| Independent Pipelines Ltd | Oil | All Hampshire districts | 0 |
| Independent Power Networks Ltd | Power | All Hampshire districts | 0 |
| Indigo Pipelines14 | Power | Unknown | 0 |
| Internal Communication Systems Ltd | Telecom | All Hampshire districts | 0 |
| Internetty.uk | Telecom | Rushmoor | 0 |
| Last Mile Asset Management | Power | Unknown | 0 |
| Last Mile Electricity Ltd8 | Power | All Hampshire Districts | 31 |
| Level 3 Communications Ltd13 | Telecom | All Hampshire districts | 12 |
| Lumen Technologies13 | Telecom | All Hampshire Districts | 12 |
| M.O.D | Other Authority | All Hampshire districts | Unknown |
| Murphy Power Distribution | Power | New Forest | 1 |
| National Grid | Power | All Hampshire districts | 68 |
| Neos Networks15 | Telecom | All Hampshire Districts | 66 |
| NEOSCORP Ltd15 | Telecom | All Hampshire districts | 66 |
| Network Rail | Other Authority | All Hampshire districts | N/A |
| Newnet plc | Telecom | All Hampshire districts | 0 |
| Nextgenaccess Ltd | Telecom | Rushmoor | 1 |
| NTL Telewest16 | Telecom | All Hampshire districts | 217 |
| NWP Street Ltd | Telecom | Unknown | 0 |
| O217 | Telecom | All Hampshire districts | 6 |
| Oil & Pipeline Agency | Oil | All Hampshire districts | 0 |
| Optical Fibre Infrastructure Ltd | Telecom | Basingstoke | 0 |
| Orange Personal Communications | Telecom | All Hampshire districts | 0 |
| Portsmouth Water | Water | East Hants, Fareham, Gosport, Havant, Winchester | 1889 |
| Redstone Network Services | Telecom | All Hampshire districts | 0 |
| ROMEC | Postal | All Hampshire districts | 24 |
| SGN | Gas | All Hampshire districts | 2614 |
| Sky Telecommunications Services Ltd4 | Telecom | Basingstoke, Test Valley | 2 |
| South East Water | Water | Basingstoke, East Hants, Hart, Rushmoor | 3959 |
| South West Water2 | Water | New Forest, Test Valley | 817 |
| Southern Water | Water | Basingstoke, Eastleigh, Fareham, New Forest, Winchester, Test Valley | 5825 |
| SSE | Power | All Hampshire districts | 2943 |
| SSE Telecom15 | Telecom | All Hampshire districts | 66 |
| Swish Fibre | Telecom | East Hants | 0 |
| T Mobile | Telecom | All Hampshire districts | 141 |
| Thames Water | Telecom | Basingstoke, East Hants, Hart, Rushmoor, Winchester | 474 |
| Thus | Telecom | All Hampshire districts | 0 |
| Toob Ltd | Telecom | All Hampshire districts | 1 |
| Torch Communications | Telecom | All Hampshire districts | 0 |
| Trooli Ltd18 | Telecom | All Hampshire Districts | 2 |
| UK Power Distribution Ltd | Power | Test Valley | 1 |
| Utility Grid Installations | Power / Water | All Hampshire districts | 0 |
| Verizon | Telecom | All Hampshire districts | 0 |
| Virgin Media | Telecom | All Hampshire districts | 3915 |
| Vodafone5 | Telecom | All Hampshire districts | 217 |
| Veolia Water Outsourcing | Water | Rushmoor | 7 |
| Vtesse Networks | Telecom | All Hampshire districts | 0 |
| Wessex Water Services | Water | New Forest | 39 |
| Zayo Group UK Ltd1 | Telecom | Basingstoke, Rushmoor | 4 |
| Zzoomm Plc | Telecom | Hart | 0 |

- Abovenet purchased by Zayo Group

- Bournemouth Water now operating as South West Water

- British Gas connections known as ESP Connections

- BSkyB operating as Sky Telecommunications

- Cable and Wireless purchased by Vodafone

- City of London Telecoms also known as COLT

- Concept Solutions operating as Glide Fibre

- Energetics operating as Last Mile

- Energis became a subsidiary of Cable and Wireless

- Energy Assets Networks Ltd also known as Utility Distribution Networks

- Giganet previously known as M12 Solutions

- Glide Business previously known as Warwicknet Ltd

- Lumen previously known as Global Crossing, Century Link Comms, Level 3 Communications

- Indigo Pipelines previously known as SSE Pipelines

- Neos Networks previously known as Neoscorp, SSE Telecoms, SSE Datacom

- NTL also known as Cable and Wireless

- O2 previously known as Telefonica

- Trooli previously known as Callflow Solutions