S278 Healthy Streets Checklist

| Project: |  | HDP Reference: | 6/ |
| --- | --- | --- | --- |
| Developer: |  | MasterGov Ref: |  |
| Developer’s Design Consultant: |  | HDP Officer: |  |
| HCC Project No. | R.J or C.J |  |  |

|  | Pre-Planning / Planning |  |  | Detailed Design |  |  | Substantial Completion |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Is there? | Incl. in design? | If not included in design, provide reasoning | Is there? | Incl. in design? | If not included in design, provide reasoning | Incl. in design? | If not completed, provide reasoning |
| Missing or incorrect tactile paving |  |  |  |  |  |  |  |  |
| Missing dropped kerb |  |  |  |  |  |  |  |  |
| Dropped kerb that can be moved to desire-line |  |  |  |  |  |  |  |  |
| Opportunity to narrow side road entrance and/or tighten priority junction corner radii |  |  |  |  |  |  |  |  |
| Assess whether appropriate to have raised/side road treatment in accordance with TG2 and 10 |  |  |  |  |  |  |  |  |
| Opportunity to narrow/raise cross-over |  |  |  |  |  |  |  |  |
| Redundant dropped kerb to remove |  |  |  |  |  |  |  |  |
| Crossing point (away from junction) that could be raised table. Not appropriate on P1 salt route or primary street. |  |  |  |  |  |  |  |  |
| Opportunity to reduce the gradient of existing crossing points that have overly steep gradients |  |  |  |  |  |  |  |  |
| Missing section of footway |  |  |  |  |  |  |  |  |
| Opportunity to increase the clear level width for walking on the footway to at least 2m wide |  |  |  |  |  |  |  |  |
| Opportunity to increase the clear level width for cycling to be in accordance with LTN1/20 |  |  |  |  |  |  |  |  |
| Signage that could be removed, resized, consolidated, de-illuminated |  |  |  |  |  |  |  |  |
| Signage that could be repositioned |  |  |  |  |  |  |  |  |
| Bollard /guard railing that could be removed. To be assessed by Road Safety Auditor familiar with “Healthy Streets” to confirm not required for safety. |  |  |  |  |  |  |  |  |
| Bollard /guard railing that could be replaced with a Sheffield stand, seat or planter |  |  |  |  |  |  |  |  |
| Inset parking/loading bay that could be raised |  |  |  |  |  |  |  |  |
| Redundant access/crossover that can be reinstated. Consultation with HDP required. |  |  |  |  |  |  |  |  |
| Opportunity to add seat (ideally maximum of 100m between seating) |  |  |  |  |  |  |  |  |
| Opportunity to add tree |  |  |  |  |  |  |  |  |
| Opportunity to add cycle stand (ideally maximum of 100m between cycle stands) |  |  |  |  |  |  |  |  |
| Opportunity to widen footway |  |  |  |  |  |  |  |  |
| Phone box that could be removed |  |  |  |  |  |  |  |  |
| Opportunity to improve accessibility to a bus stop |  |  |  |  |  |  |  |  |
| Opportunity to add a shelter/seat at a bus stop |  |  |  |  |  |  |  |  |
| Road markings that could be removed |  |  |  |  |  |  |  |  |
| Could any speed cushions be replaced with a raised table to facilitate crossing (provided there is adequate visibility) |  |  |  |  |  |  |  |  |
| Footway/cycleway that could be levelled or footway defects that could be addressed |  |  |  |  |  |  |  |  |
| Parking/loading bays that could be removed or reconfigured |  |  |  |  |  |  |  |  |
| Opportunity to add buildouts to slow traffic, widen footways, accommodate growth of mature trees |  |  |  |  |  |  |  |  |
| Gully cleaning/drainage improvements |  |  |  |  |  |  |  |  |
| Temporary clutter to be removed e.g. a-boards/fly tipped rubbish |  |  |  |  |  |  |  |  |
| Broken street light to be repaired |  |  |  |  |  |  |  |  |
| Opportunity to incorporate a controlled crossing facility to any appropriate arms (subject to need/traffic flow – Refer to TG10 and TG4-2) |  |  |  |  |  |  |  |  |

Additional actions or comments Text here.